TRACK RECORDING RESULTS (CHORD MODE)

| TRC No: TRC-8003 | Directorate: TMM | Software Version: 34.2.0 date: 12.07.2026 |
| --- | --- | --- |
| Railways: DFC | Division: EDFC | Section: SNL- UBCD |
| Date of Rec: 24.08.2026 | Route Speed: 100 kmph | Sampling Distance Actual: 0.25 M |
| AL Filter: 3-70 Metre | UN Filter: 3-70 Metre | Sampling Distance Entered: 0.25 M |

PARAMETER SETTINGS

|  | UN1 | UN2 | AL1 | AL2 | TWS1 | AV-G-S | AV-G-CM | AV-G-CL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chord/Base(Metre) | 9 | 18 | 9 | 15 | 3 |  |  |  |
| Route Speed <= 100 kmph |  |  |  |  |  |  |  |  |
| SD New Track Limits (mm) NTL | 2 | --- | 1.4 | --- | --- | --- | --- | --- |
| SD Maintenance Limits (mm) PML | 5 | --- | 3.3 | --- | --- | --- | --- | --- |
| SD/AVG Maintenance Limits (mm) NBML | 6.8 | --- | 4.9 | --- | --- | -8/+10 | -5/+14 | -5/+18 |
| AVG Maintenance Limits (mm) UML | --- | --- | --- | --- | --- | -10/+12 | -7/+17 | -7/+20 |

PEAK LIMITS (mm)

|  | Short Chord |  |  |  |  |  | Long Chord |  | Acceleration |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UN1 | AL1 | TWS1 | AGS | AGC-M | AGC-L | UN2 | AL2 | Vertical | Lateral |
| Route Speed <= 100 kmph |  |  |  |  |  |  |  |  |  |  |
| New Track Limit (NTL) | 6 | 4 | --- | --- | --- | --- | --- | --- | --- | --- |
| Need Based Maintenance Limit (NBML) | 20 | 15 | 15 | -10/+12 | -7/+17 | -6/+22 | --- | --- | --- | --- |
| Urgent Maintenance Limit (UML) | --- | --- | 21 | -12/+15 | -11/+20 | -8/+25 | --- | --- | 0.3g | 0.3g |

TWIST CALCULATED FROM SUPER ELEVATION

TRACK RECORDING RESULTS (PROFILE MODE)

| TRC No: TRC-8003 | Directorate: TMM | Software Version: 34.2.0 date: 12.07.2026 |
| --- | --- | --- |
| Railways: DFC | Division: EDFC | Section: SNL- UBCD |
| Date of Rec: 24.08.2026 | Route Speed: 100 kmph | Sampling Distance Actual: 0.25 M |
| AL Filter: 3-70 Metre | UN Filter: 3-70 Metre | Sampling Distance Entered: 0.25 M |

PARAMETER SETTINGS

|  | UN1 | UN2 | AL1 | AL2 | TWS1 | AV-G-S | AV-G-CM | AV-G-CL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chord/Base(Metre) | 3-25 | 25-70 | 3-25 | 25-70 | 3 |  |  |  |
| Route Speed <= 100 kmph |  |  |  |  |  |  |  |  |
| SD New Track Limits (mm) NTL | 2 | --- | 1.4 | --- | --- | --- | --- | --- |
| SD Maintenance Limits (mm) PML | 5 | --- | 3.3 | --- | --- | --- | --- | --- |
| SD/AVG Maintenance Limits (mm) NBML | 6.8 | --- | 4.9 | --- | --- | -8/+10 | -5/+14 | -5/+18 |
| AVG Maintenance Limits (mm) UML | --- | --- | --- | --- | --- | -10/+12 | -7/+17 | -7/+20 |

PEAK LIMITS (mm)

|  | Short Chord |  |  |  | Long Chord |  |  |  | Acceleration |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | UN1 | AL1 | TWS1 | AGS |  | AGC-M | AGC-L | UN2 |  | AL2 | Vertical | Lateral |
| Route Speed <= 100 kmph |  |  |  |  |  |  |  |  |  |  |  |  |
| New Track Limit (NTL) | 6 | 4 | --- | --- |  | --- | --- | --- |  | --- | --- | --- |
| Need Based Maintenance Limit (NBML) | 20 | 15 | 15 | -10/+12 |  | -7/+17 | -6/+22 | --- |  | --- | --- | --- |
| Urgent Maintenance Limit (UML) | --- | --- | 21 | -12/+15 |  | -11/+20 | -8/+25 | --- |  | --- | 0.3g | 0.3g |

TWIST CALCULATED FROM SUPER ELEVATION

VEHICLE PARAMETER RESULTS

| TRC No: TRC-8003 | Directorate: TMM | Software Version: 34.2.0 date: 12.07.2026 |
| --- | --- | --- |
| Railways: DFC | Division: EDFC | Section: SNL- UBCD |
| Date of Rec: 24.08.2026 | Route Speed: 100 | Sampling Distance Actual: 0.25 |
| AL Filter: 3-70 | UN Filter: 3-70 | Sampling Distance Entered: 0.25 |

| PARAMETER SETTINGS RIDE INDEX AND RMS THRESHOLDS |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ROUTE SPEED <= 100 KM/H |  |  |  |  |  |  |  |  |  |  |
| LIMITS | RIDE INDEX |  |  |  |  | RMS |  |  |  |  |
|  | TRC-PIVOT |  | TEST VEHICLE |  | AXLE BOX |  | TRC-PIVOT |  | TEST VEHICLE |  |
|  | VERTICAL | LATERAL | VERTICAL | LATERAL | VERTICAL | LATERAL | VERTICAL | LATERAL | VERTICAL | LATERAL |
| P1 | 3.5 | 3.5 | 3.75 | 3.75 | 0.8 | 0.8 | 0.3 | 0.3 | 0.35 | 0.35 |
| P2 | 3.75 | 3.75 | 4 | 4 | 0.9 | 0.9 | 0.35 | 0.35 | 0.4 | 0.4 |
| P3 | 4 | 4 | 4.25 | 4.25 | 1 | 1 | 0.4 | 0.4 | 0.5 | 0.5 |

ACCELERATION THRESHOLD

| PEAK LIMITS | AXLE BOX |  | TRC-PIVOT |  | TEST VEHICLE |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | VERTICAL | LATERAL | VERTICAL | LATERAL | VERTICAL | LATERAL |
| P1 | 2 | 1 | 0.3 | 0.3 | 0.35 | 0.35 |
| P2 | 4 | 2 | 0.35 | 0.35 | 0.4 | 0.4 |
| P3 | 7 | 4 | 0.4 | 0.4 | 0.45 | 0.45 |